# Department of Computer Science and Engineering

| Course Code: | Credits: 1 |
| --- | --- |
| Course Name: Microprocessors | Semester: |

Lab 04

Flow control instructions and Branching Structures

## Topic Overview:

For assembly language-programs to carry out useful tasks, there must be a way to make decisions. In this lab, students will familiarize themselves with how decisions can be made with the jump instruction. The jump instructions transfer control to another part of the program. This transfer can be unconditional or can depend on a particular combination of status flag settings. After introducing the jump instructions, we'll use them to implement high-level language decision structures.

## Lesson Fit:

There is prerequisite to this lab. Students must have a basic idea on the following concepts:

  - Flag register

  - Some basic operations such as MOV, ADD, SUB, MUL and DIV

  - Basic I/O operations

  - Character encoding using ASCII

## Learning Outcome:

After this lecture, the students will be able to:

  - Control flow of the program

  - Write conditional statements in assembly

## Anticipated Challenges and Possible Solutions

  - Students may find it difficult to visualize how the program control flow changes when using jump

## Solutions:

    - Step by step simulation

  - Directly coding in assembly may come off as challenging.

## Solutions:

    - Writing the pseudocode first then then converting it to assembly may help

- Activity Detail

  - Hour: 1

Discussion: Jump instruction

## Conditional Jumps:

if (x>5){

// code

}

In the line if (x>5), a comparison is done between the content of x and 5. The decision whether to execute the enclosed code or not depends on the result of the comparison. In assembly the comparison is done by a piece of code called CMP. The syntax of CMP is CMP destination, source. The comparison is done by destination - source. The result is not saved anywhere but affects the flags. The result of subtraction can be 0, AX > BX or AX< BX. Below is a piece of code.

## MOV AX, first_number MOV BX, second_number CMP AX, BX

The comparison is done in the third line. Now there could be 5 possibilities AX==BX, AX> BX, AX< BX, AX>= BX, AX<= BX. We will take the decision based on one of these options. This decision is performed by "jump" instruction denote as "j".

| Condition | Jump Instruction | Explanation |
| --- | --- | --- |
| AX == BX | JE | jump if destination and source are equal |
| AX > BX | JG | jump if destination > source |
| AX < BX | JL | jump if destination < source |
| AX >= BX | JGE | jump if destination >= source |
| AX <= BX | JLE | jump if destination <= source |

## But jump where???

Jump in that line which we want to execute when one of the above conditions is satisfied. How will we get the line number? We do not have to worry about the line numbers because we will name the line(s) ourselves.

| int x = 5; | mov ax, 5 cmp ax, 4 JG My_Line |
| --- | --- |
| if (x > 4) { //code } | My_Line: ;code |

Node the declaration of the line name ends with a colon (:).

## Please go through page number 96 of the book to know about more conditional jumps.

Unconditional jump

For skipping a portion of code, we use/ need unconditional jump. There is no comparison, it is just a jump from one line to the one we want. The instruction is called JMP and its syntax is JMP destination line_name.

mov ax,5 mov bx,8

jmp my_line mov ah,4 mov dl,6 my_line

mov dl,7

In the above code, when the 3rd line is executing the program jumps to the line named my_line without executing the codes in between, This is how codes are skipped.

# Branching Structures

In high-level languages, branching structures enable a program to take different paths, depending on conditions. In this section, we'll look at three structures.

## IF-THEN :

The IF-THEN structure may be expressed in pseudo code as follows:

The condition is an expression that is true or false. If it is true, the true-branch statements are executed. If it is false, nothing is done, and the program goes on to whatever follows.

Example: Replace the number in AX by its absolute value Solution: A pseudocode algorithm is:

IF AX < 0

THEN replace AX by -AX END_IF

It can be coded as follows:

The condition AX < 0 is expressed by CMP AX,0. If AX is not less than 0, there is nothing to do, so we use a JNL (jump if not less) to jump around the NEG AX. If condition AX < 0 is true, the program goes on to execute NEG AX.

Problem Task: Task 01 (Page 8)

## Hour: 2

Discussion: Branching Structures (Cont.) IF-THEN-ELSE:

The jmp instruction comes in use when if - else condition is employed. Let us see the use of conditional and unconditional jumps together in a program where we will find the greater of 2 numbers.

# JAVA

if (x>y){

System.out.println("enter the second number"); int y = k.nextInt();

System.out.println("enter the first number"); int x = k.nextInt();

System.out.println(x+" is greater");

}

else{

System.out.println(y+" is greater");

ASSEMBLY:

data segment

; add your data here!

pkey db "press any key...$"

- db "Enter first number$"

- db "Enter second number$" c db " is larger$"

ends

stack segment dw 128 dup(0)

ends code segment start:

; set segment registers: mov ax, data

mov ds, ax mov es, ax

; add your code here

lea dx, a ;Print a mov ah,9

int 21h

mov ah,1 int 21h

mov bl,al ;move input to bl

lea dx, b ;Print b mov ah,9

int 21h

mov ah,1 int 21h

mov dl,bl mov ah,2 int 21h

lea dx, c mov ah,9 int 21h jmp

line1:

mov dl,cl mov ah,2 int 21h

lea dx, c mov ah,9 int 21h

e:

lea dx, pkey mov ah, 9

int 21h ;output string ds:dx

;wait for any key.... mov ah, 1

int 21h

mov	ax,	4c00h	;exit operating

system. int 21h

ends

end start ; set entry point stop the assembler.

## Discussion on CASE

A CASE is a multiway branch structure that tests a register, variable, or expression for particular values or a range of values. The general form is as follows:

CASE expression Values _ 1: statements_1 Values _ 2: statements_2

..

..

Values_n: statements_n END_CASE

In this structure, expression is tested; if its value is a member of the set values_1, then statements_1 are executed. We assume that sets values_l, … , values_n are disjoint

## Hour: 3

Discussion:

Check progress while the students carry on with the rest of the tasks.

Problem Task: Task 10-14 (Page 9)

- Home Tasks: All the unfinished lab tasks