# Department of Computer Science and Engineering

| Course Code: | Credits: 1 |
| --- | --- |
| Course Name: Microprocessors | Semester: Fall’24 |

Lab 05

Flow control instructions and Looping structures

## Topic Overview:

Alongside the capability of making decisions, any functional program should also have a mechanism to repeat sections of code. In this lab, students will familiarize themselves with the loop instructions to achieve that. Alongside jump, the loop instruction is also used to transfer control to another part of the program. The students will then learn to implement different looping structures. This application will make it much easier to convert a pseudo code algorithm to assembly code.

## Lesson Fit:

There is prerequisite to this lab. Students must have a basic idea on the following concepts:

  - Jump instruction

  - Some basic operations such as MOV, ADD, SUB, MUL and DIV

  - Basic I/O operations

  - Character encoding using ASCII

## Learning Outcome:

After this lecture, the students will be able to:

  - Control flow of the program

  - Use loops to avoid repetition

## Anticipated Challenges and Possible Solutions

  - Students may find it difficult to visualize how the program control flow changes when using loop/jump

## Solutions:

    - Step by step simulation

  - Directly coding in assembly may come off as challenging

## Solutions:

    - Writing the pseudocode first then then converting it to assembly may help

## Activity Detail

  - Hour: 1

## Discussion: Looping Structure

Just like for and while loops in high level programming languages, loops can also be implemented in assembly. There are 2 ways of implementing a loop: explicit and implicit.

- Explicit: By using compare and jump instructions to decide whether to enter the loop or not and by using the inc /add /sub instruction for increments.

# Java

int x = 0;

while (x < 5){	 System.out.println(x);

x++;

}

# Assembly

mov ah,2

start:

cmp dl,35h ;the emu8086 uses hexadecimal. 35h is 5

jge end

jmp start

end:

inc dl

int 21h

mov dl,30h ;the emu8086 uses hexadecimal. 30h is 0

- Implicit: In this case we do not have to check whether the counter has reached the limit or not. This will be done automatically. The instructions will be loop. loop destination_line is the syntax. CX will be used as the counter always in order for the loop instruction to execute.

# JAVA

int x = 0; while (x < 5){

System.out.println(x); x++;

}

# Assembly

mov cx,5 ;the bound will be in cx. (the number of times the loop will run)

mov dl,30h mov ah,2 start:

int 21h inc dl

loop start

NB. CX will always start from the specified count and will always decrement by 1.

Problem Task: Task 01-Task03 (Page 5)

## Hour: 2

Discussion: Discuss the properties of a repeat-until loop structure.

- Repeat Until loop: Repeat the loop until a condition is satisfied. For example you have been asked to take in characters from the user and print them until a space is pressed.

.code repeat:

mov ah,1 int 21h mov ah,2 mov dl,al int 21h cmp al,' ' jne repeat

Problem Task: Task 04 – 06 (Page 5-6)