8086 Flag Register

Dept. of Computer Science and Engineering

Lecture References:

□ Book:

□	Microprocessors and Interfacing: Programming and Hardware, Chapter # 2, Author: Douglas V. Hall

8086 Flag Register

- 16-Bit register

  - 7 bits are undefined/unused (marked by red x in the figure below)

  - 6 status/condition flags (marked by blue circles)

  - 3 control flags (those in grey boxes)

- The condition flags are set (1) or reset (0) depending on the result of an arithmetic/logical operation.

- Control flags control the operations of the CPU

Bit Positions

15	14	13	12	11	10	9	8	7	6	5	4	3	2	1	0

# Status Flags

Flags	are	useful	in	programming	e.g.	when	writing conditions such as:

- If answer is zero, do … else …. // zero flag comes in hand here

- If  answer is less than zero, do … else …. // sign flag can be

4	used here

# Status Flags

- Carry Flag (CF) - is set to ’1’ when there is an unsigned overflow. E.g. when you add bytes 255 + 1 (result is not in range

0...255). When there is no overflow this flag is reset to 0.

- Parity Flag (PF) - set to ’1’ when there is even number of one bits in result, and reset to ’0’ when there is odd number of one bits. Even if result is a word only lower 8 bits are analyzed!

- Auxiliary Flag (AF) - set to ’1’ when there is an unsigned overflow for low nibble (4 bits).

# Status Flags

- Zero Flag (ZF) - set to ’1’	when result is zero. For non-zero result this flag is reset to ’0’.

- Sign Flag (SF) - set to ’1’ when result is negative. When result is positive it is reset to ‘0’. (This flag takes the value of the most

significant bit).

- Overflow Flag (OF) - set to ’1’ when there is a signed overflow. For example, when you add bytes 100 + 50.

NB:

OF is set to 1 if there is a carry from:

- the 7th bit to the 8th bit ONLY or

- from the 8th bit to the 9th bit ONLY

If there is a carry from the 7th bit to the 8th bit and from the 8th bit to the 9th bit at THE SAME TIME then OF = 0

Status Flags

OF=1

Set because there is a carry from the 7th bit to the 8th bit ONLY

Carry	1	1	1

AF=0

Reset because there is NO carry from the lower nibble to the upper nibble

  - MOV AL, 50h  ( 0 1 0 1 0 0 0 0 )

  - MOV BL, 32h  ( 0 0 1 1 0 0 1 0 )

- ADD AL, BL

(x 1 0 0 0 0 0 1 0 )

CF=0	ZF=0

Reset because the answer has NO carry

SF=1

Set	because	the	MSB	is	1 indicating a negative answer

Reset because the answer is NOT zero

PF=1

Set because the answer has an EVEN number of 1s

NB:

OF is reset to 0 if:

- there is a carry from the the 7th bit to the 8th bit and from the

8th bit to the 9th bit at THE SAME TIME or

- there is NO carry from the the 7th bit to the 8th bit and from the 8th bit to the 9th bit at THE SAME TIME

Status Flags

OF=0

Reset because there is a carry from the 7th bit to the 8th bit and from the 8th bit to the 9th bit at THE SAME TIME

Carry	1	1	1	1	1	1	1	1

AF=1

Set because there IS a carry from

| - MOV | AL, | FFh | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | the lower nibble to the upper nibble |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| - MOV | BL, | 01h | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 |  |

  - ADD AL, BL

CF=1

Set  because  the  answer  has  a

1 0 0 0 0 0 0 0 0

ZF=1

Set because the answer IS zero

PF=1

carry	SF=0

Reset	because	the	MSB	is	0 indicating a positive answer

Set because the answer has an EVEN number of 1s

# Control Flags:

Control Flags:

  - Trap Flag (TF) - Used for on-chip single-step debugging.

  - Interrupt enable Flag (IF) - when this flag is set to ’1’ CPU reacts to interrupts from external devices.

  - Direction Flag (DF) - this flag is used by some instructions to process data chains, when this flag is set to ’0’ - the processing is done forward, when this flag is

set to ‘1’ the processing is done backward.

Quiz: Status Flag Values?

| - MOV | AX, | ABCDh |
| --- | --- | --- |
| - MOV | BX, | 9876h |
| - ADD | AX, | BX |

Thank You !!